CCT member and observer Activity Report

Period: from January to December 2025

Institute: Centro Nacional de Metrología (CENAM)

State economy: México

Number of persons involved in thermometry of the institute: 9

The Thermometry Group (DTR) maintains, as a National Temperature Standard, the reproduction of the International Temperature Scale of 1990 (EIT-90). In addition, the DTR maintains the National Standards of Humidity in Air, Moisture in Solids, Thermal Conductivity of Non-Conductive Solids, Thermal Conductivity of Conductive Solids, and Thermal Conductivity of Simple Fluids.

DTR has started research work in primary thermometry based on Doppler-induced broadening, in conjunction with the time and frequency group of CENAM and the Center for Advanced Studies, CINVESTAV – National Polytechnic Institute.

DTR staff collaborate directly with staff from working groups of other National Metrology Institutes around the world, and have carried out training stays, the development of joint projects, and comparisons of measurement results.

The dissemination of the values of the national standards is carried out through the calibration of the standards of the secondary laboratories. With this, secondary laboratories and industry in general can reach the levels of uncertainty in their measurements suitable for their processes.

Summary of research and development:

Started research work on primary thermometry based on the measurement of the Doppler-induced broadening of a well-isolated absorption line in a diluted gas in thermodynamic equilibrium, using laser absorption spectroscopy in the linear regime of light–matter interaction. The Doppler width reflects the Maxwell-Boltzmann velocity distribution of gas particles and relates the thermal energy kBT to an optical frequency ν, reducing temperature determination to a frequency measurement—the most precisely measurable quantity. Leveraging this direct frequency-temperature relationship, we began developing an experimental setup to measure the absolute thermodynamic temperature of 133Cs atomic gas using laser spectroscopy on the D2 line, thereby achieving a practical realization of the kelvin through Doppler broadening thermometry.

Development of a measuring model for the gravimetric method by drying in N-stages applied to materials with high moisture content. The gravimetric method by oven drying is widely used for moisture measurement in solid materials, following a drying protocol that includes specific conditions such as drying time and temperature. Although this method is reliable for many types of materials, some materials with high moisture content require a pre-drying process at low temperature before final drying. In these cases, the drying process can be carried out in several stages to achieve favorable grinding conditions.

Have continued work on fixed points for radiation thermometry below the silver point:

(1) Design and construction of fixed-point blackbodies for the calibration of radiation thermometers below the silver point: Tin freeze point blackbody. Presently, we have the following fixed points below the silver point: aluminium, indium, gallium, and eutectic indium-gallium.

(2) Design and construction of low-temperature furnaces for fixed-point blackbodies below the silver point.

Started a project to design, build, and evaluate a measurement system that allows the verification and calibration of thermometers used to measure air temperature.

Summary of recent comparison activity:

Collaboration on Technical Protocol and preparation for the SIM.T-K6 and SIM.T-K8 Comparison of Realizations of Dew/Frost-Point Temperature over the Range - 30 °C to + 65 °C. The participants’ results for the below ambient dew/frost-point temperatures: - 30 °C, - 10 °C, + 1 °C and + 20 °C, will be linked to CCT-K6 Key Comparison Reference Value (KCRV), and results for the following above ambient dew point temperatures: + 30 °C, + 50 °C and + 65 °C, will be linked to CCT-K8 Key Comparison Reference Value (KCRV), in which the National Institute of Standards and Technology (NIST) previously participated. The completed registration form for the proposed SIM.T-K6 dew/frost-point temperature intercomparison and SIM.T-K8 dew-point temperature intercomparison was submitted by the pilot (NITS) on 07/01/2026 and registered in the KCDB database on 15/01/2026.

Summary of other activities:

Support to SIM-NMI

for humidity measurements.

for moisture content in grains (mainly corn).

for the National Organizations in charge of the cold chain in medicines and food.

National Health Sector

For the State Health Laboratories. Training and consulting on temperature measurement with clinical thermometers and strengthening of their calibration capabilities.

For CQMS (Querétaro State Medical and Health Cluster). Pilot study of clinical thermometers.

For COFEPRIS (Federal Commission for Protection against Sanitary Risks). Development of test methods for clinical thermometers and support in strengthening national regulations.

National Food Sector

For the CANAMI (National Chamber of Industrialized Corn). Development of moisture content measurements in corn samples to verify their commercial meters. Promoting the implementation of the national standard NOM-SECOFI-226-2018.

For APEAM (Mexican Association of Avocado Producers). Characterization of the dry matter measurement system using the microwave method and comparison with the method developed at CENAM (National Center for Metrology). Development of a calibration method for meters used to measure dry matter in Hass avocados.

For BUAP (Puebla State University). Characterization of the soil moisture content measurement system for use in calibrating commercial meters (including the BUAP prototype). Development of an infrared thermography calibration method for agricultural applications related to the health of plants and vegetables

National Climate Change Sector

For SEMARNAT (Ministry of Environment and Natural Resources) and the National Meteorological Service. Development of a calibration system for automatic weather stations.

List of bibliography (last 5 years):

1. The Influence of Average Effective Emissivity in the Determination of Radiance Temperature of a Surface, D. Cywiak-Córdova, D. Cárdenas-García, E. Martines-López, H. Rodríguez-Arteaga, MAPAN-Journal of Metrology Society of India, https://doi.org/10.1007/s12647-025-00806-0, 2025.

2. Measuring Model for the Gravimetric Method by Drying in N-Stages Applied to Materials with High Moisture Content, E. Martines-López, D. Cárdenas-García, D. Cywiak-Córdova, MAPAN-Journal of Metrology Society of India, https://doi.org/10.1007/s12647-023-00709-y, 2024.

3. INTERPRETACIÓN Y USO DE LOS RESULTADOS DE UN CERTIFICADO DE CALIBRACIÓN DE TERMÓMETRO DE RESISTENCIA DE PLATINO, Martines López E., Cárdenas García D., Cywiak Cordova D., MEMORIAS SM 2022, ISBN 978-607-98045-3-4, Querétaro, México, febrero 2024.

4. MODELO DE MEDICIÓN Y ANÁLISIS DE INCERTIDUMBRE EN LA CALIBRACIÓN DE TERMÓMETROS DE CONTACTO DE LECTURA DIRECTA, D. Cywiak Córdova, E. Martines López, D. Cárdenas García, MEMORIAS SM 2022, ISBN 978-607-98045-3-4, Querétaro, México, febrero 2024.

5. MODELO DE MEDICIÓN Y ANÁLISIS DE INCERTIDUMBRE EN LA CALIBRACIÓN DE TERMÓMETROS DE RESISTENCIA DE PLATINO POR COMPARACIÓN, D. Cywiak Córdova, E. Martines López, D. Cárdenas García, MEMORIAS SM 2022, ISBN 978-607-98045-3-4, Querétaro, México, febrero 2024.

6. EMISIVIDAD EFECTIVA PROMEDIO Y SU EFECTO EN LA DETERMINACIÓN DE LA TEMPERATURA DE RADIANCIA DE UNA SUPERFICIE OPACA, Hugo Rodríguez Arteaga, Daniel Cárdenas García, David Cywiak Córdova, Verónica Sastre Muñoz, MEMORIAS SM 2022, ISBN 978-607-98045-3-4, Querétaro, México, febrero 2024.

7. EL MODELO DE MEDICIÓN DE EMISIVIDAD ESPECTRAL DEL CENAM, Daniel Cárdenas García y Enrique Martines López, MEMORIAS SM 2022, ISBN 978-607-98045-3-4, Querétaro, México, febrero 2024.

8. TERMOMETRÍA DE RADIACIÓN EN EL CENAM: EL PATRÓN NACIONAL DE TEMPERATURA ARRIBA DEL PUNTO DE SOLIDIFICACIÓN DE LA PLATA, Daniel Cárdenas García, Enrique Martines López y David Cywiak Córdova, MEMORIAS SM 2022, ISBN 978-607-98045-3-4, Querétaro, México, febrero 2024.

9. PATRÓN NACIONAL DE CONDUCTIVIDAD TÉRMICA DE SÓLIDOS NO CONDUCTORES CNM-PNE-16, O. González-Rodríguez, García Duarte S., Lira-Cortés L., MEMORIAS SM 2022, ISBN 978-607-98045-3-4, Querétaro, México, febrero 2024.

10. MODELO DE MEDICIÓN PARA MATERIALES SÓLIDOS CON ALTO CONTENIDO DE HUMEDAD, Enrique Martines López, Daniel Cárdenas García, MEMORIAS SM 2022, ISBN 978-607-98045-3-4, Querétaro, México, febrero 2024.

Conferences

1. CALIBRATION OF REFERENCE BLACKBODIES FOR TYMPANIC THERMOMETERS USING AN INDUSTRIAL ROBOT, I. Mueller, O. Bothur, D. Pohlandt, D. Cárdenas García, TEMPMEKO ISHM SYMPOSIUM 2025, 20-24 October 2025, Reims, France.

2. TOWARDS LASER-BASED PRIMARY THERMOMETRY: LINKING THE THERMODYNAMIC TEMPERATURE OF ATOMIC VAPORS TO AN OPTICAL FREQUENCY, Sebastián Martínez Sánchez, Verónica Nieto Peralta, Alexis Arturo Villareal Wong, Eduardo Issac Vidal Delgado, Cinthia Figueroa Carrera, Leonel Lira Cortés, Carlos Andrés Ortiz Cardona, Eduardo de Carlos López, José Mauricio López Romero, 33rd International Materials Research Congress 2025, 17-21 August 2025, Cancún, México.

3. INTERPRETACIÓN Y USO DE LOS RESULTADOS DE UN CERTIFICADO DE CALIBRACIÓN DE TERMÓMETRO DE RESISTENCIA DE PLATINO, Martines López E., Cárdenas García D., Cywiak Córdova D., Simposio de Metrología 2022, 26-28 de octubre de 2022, Querétaro, México.

4. MODELO DE MEDICIÓN Y ANÁLISIS DE INCERTIDUMBRE EN LA CALIBRACIÓN DE TERMÓMETROS DE CONTACTO DE LECTURA DIRECTA, D. Cywiak Córdova, E. Martines López, D. Cárdenas García, Simposio de Metrología 2022, 26-28 de octubre de 2022, Querétaro, México.

5. MODELO DE MEDICIÓN Y ANÁLISIS DE INCERTIDUMBRE EN LA CALIBRACIÓN DE TERMÓMETROS DE RESISTENCIA DE PLATINO POR COMPARACIÓN, D. Cywiak Córdova, E. Martines López, D. Cárdenas García, Simposio de Metrología 2022, 26-28 de octubre de 2022, Querétaro, México.

6. EMISIVIDAD EFECTIVA PROMEDIO Y SU EFECTO EN LA DETERMINACIÓN DE LA TEMPERATURA DE RADIANCIA DE UNA SUPERFICIE OPACA, Hugo Rodríguez Arteaga, Daniel Cárdenas García, David Cywiak Córdova, Verónica Sastre Muñoz, Simposio de Metrología 2022, 26-28 de octubre de 2022, Querétaro, México.

7. EL MODELO DE MEDICIÓN DE EMISIVIDAD ESPECTRAL DEL CENAM, Daniel Cárdenas García y Enrique Martines López, Simposio de Metrología 2022, 26-28 de octubre de 2022, Querétaro, México.

8. TERMOMETRÍA DE RADIACIÓN EN EL CENAM: EL PATRÓN NACIONAL DE TEMPERATURA ARRIBA DEL PUNTO DE SOLIDIFICACIÓN DE LA PLATA, Daniel Cárdenas García, Enrique Martines López y David Cywiak Córdova, Simposio de Metrología 2022, 26-28 de octubre de 2022, Querétaro, México.

9. PATRÓN NACIONAL DE CONDUCTIVIDAD TÉRMICA DE SÓLIDOS NO CONDUCTORES CNM-PNE-16, O. González-Rodríguez, García Duarte S., Lira-Cortés L., Simposio de Metrología 2022, 26-28 de octubre de 2022, Querétaro, México.

10. MODELO DE MEDICIÓN PARA MATERIALES SÓLIDOS CON ALTO CONTENIDO DE HUMEDAD, Enrique Martines López, Daniel Cárdenas García, Simposio de Metrología 2022, 26-28 de octubre de 2022, Querétaro, México.

11. ACTIVIDADES PARA EXTENDER EL ALCANCE DEL PATRÓN NACIONAL DE HUMEDAD EN AIRE A MEDICIONES DE PUNTO DE ROCÍO O DE ESCARCHA, Jesús Alfredo Dávila Pacheco, Simposio de Metrología 2022, 26-28 de octubre de 2022, Querétaro, México.