Technical protocol for a supplementary comparison in torque

between NIS (Egypt) and PTB (Germany)

- 1 Introduction

It is recognised that for the purposes of international trade and scientific collaboration, the national measurement systems of countries around the world should agree which is the basis of the SI. The centre of each country’s national measurement system tends to be its national measurement institute (NMI), and it is important that NMIs compare their standards with each other on a regular basis. Recently, NIS built a 1 kN·m primary standard torque calibration machine to provide the highest level of the traceability pyramid from 5 N·m up to 1000 N·m in both clockwise and anticlockwise directions. NIS agreed with PTB to participate in this comparison as a pilot.

- Organization

A protocol for this supplementary comparison was issued as a result of a discussion between the National Institute of Standards (NIS),  force and material metrology laboratory and the Physikalisch-Technische Bundesanstalt (PTB), force and torque laboratory.

- Participants

| NMI | Address | Contact person |
| --- | --- | --- |
| NIS Organizer and participant | National Institute of Standards (NIS), Tersa Street, El Haram, Giza P.O. Box: 136 Giza, code 12211 Giza - Egypt. | K.M. Khaled Tel: +202 37401113 Fax: +202 33867462 Email: khaled_fmmd_nis@yahoo.com khaled.abdelaziz@nis.sci.eg |
| PTB Pilot | Physikalisch-Technische Bundesanstalt (PTB) Bundesallee 100, 38116 Braunschweig, Germany | Andreas Brüge Tel: +49 531 592-1135 Fax: +49 531 592-69-1135 Email: Andreas.bruege@ptb.de |

- Time schedule for the measurements

The comparison will be conducted during 2022, starting with the measurements at NIS and then at PTB. The following timetable is the scheduled measuring time.

Table 1. Schedule of the comparison

| Institute / Country | Time of measurements |
| --- | --- |
| NIS/Egypt | March 2022 |
| PTB/Germany | June-July 2022 |

- Traveling Standards

  - Description

A torque transducer of type TN with the capacity of 1000 N·m manufactured by HBM will be used in this comparison as an artefact. Cables and DMP40S2 measuring amplifier will be offered by NIS during the whole comparison measurements.

  - Handling

The torque transducer and the measuring amplifier will be transferred between the two institutes by airplane transport and packed in such a way to ensure robustness and to protect it from being deformed or damaged during transit.

- Method of measurement

The primary torque standard machines of NIS and PTB will be compared in accordance with the scheme given in Table 2. One torque transducer should be used to compare the machines, in conjunction with an amplifier of the highest resolution and stability. Each will be used at two torque steps to enable each pair of machines to be compared at three discrete torque levels.

Table 2. Comparison scheme

| NIS Machine | PTB Machine | Calibration Mode | Test | Torque Steps` | Transducer |
| --- | --- | --- | --- | --- | --- |
| 1000 N·m DWTSM | 1000 N·m DWTSM | Clockwise | A1 | 1000 N·m 500 N·m | 1000 N·m |
|  |  | Anti-Clockwise | A2 | -1000 N·m -500 N·m |  |

To minimize the effects of torque transducer creep, each transducer calibration will be carried out in accordance with a strict time torque profile (see Fig. 1), including the preloads which will always perform at the start of each test and after each rotation of the transducer in the machine. The time between every two successive readings is six minutes.

0֯

120֯

240֯

The rotation of the transducer should be carried out in a time interval of 10 minutes after removing the load and before taking the next zero reading (which will not be taken into account for the calculation). Three outputs will be obtained at each of three orientations, symmetrically distributed about the central axis of the machine – the deflections will be calculated by subtracting the transducer output at zero torque (prior to the application of the torque) from the transducer output under torque. An Excel sheet will be prepared for unifying the way of calculations.

- Environmental Conditions

The environmental conditions for the comparison will be the following:

- temperature: (21 ± 0.5) ºC

- relative humidity of ambient air: (40 ± 7) % RH

- Publications

The results of this comparison will be reported to the BIPM and the comparison report will be submitted to the METROLOGIA journal.

- BIBM Registration

http://kcdb.bipm.org/appendixB/KCDB_ApB_info.asp?cmp_idy=1081&cmp_cod=CCM.T-K1.3&prov=exalead,